TX0730021

2023 ANNUAL DRINKING WATER QUALITY REPORT

WEST BRAZOS WSC

OUR DRINKING WATER MEETS OR EXCEEDS ALL FEDERAL (EPA) DRINKING WATER REQUIREMENTS

This report is a summary of the quality of the water that we provide to our customers. The analysis was made by using the data from the most recent U.S. Environmental Protection Agency (EPA) required tests and is presented in the attached pages. We hope this information helps you become more knowledgeable about what is in your drinking water.

Where Do We Get Our Water? - Our drinking water is obtained from Groundwater sources. It comes from the following Aquifer: TWIN MOUNTAIN-TRAVIS PEAK.  A Source Water Susceptibility Assessment for your drinking water sources is currently being updated by the Texas Commission on Environmental Quality. This information describes the susceptibility and types of constituents that may come into contact with your drinking water source based on human activities and natural conditions. The information contained in the assessment allows us to focus our source water protection strategies. Some of this source water assessment information will be available later this year on Texas Drinking Water Watch at http://dww.tceq.state.tx.us/DWW/. For more information on source water assessments and protection efforts at our system, please contact us.

Water Sources - The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals, and in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity. Contaminants that may be present in source water before treatment include:  microbes, inorganic contaminants, pesticides, herbicides, radioactive contaminants, and organic chemical contaminants.

All drinking water may contain contaminants.  When drinking water meets federal standards there may not be any health based benefits to purchasing bottled water or point of use devices.  Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate that water poses a health risk. More information about contaminants and potential health effects can be obtained by calling EPA's Safe Drinking Water Hotline 1.800.426.4791.

Secondary Constituents - Many constituents (such as calcium, sodium, or iron) which are often found in drinking water can cause taste, color, and odor problems. The taste and odor constituents are called secondary constituents and are regulated by the State of Texas, not the EPA. These constituents are not causes for health concern. Therefore, secondaries are not required to be reported in this document, but they may greatly affect the appearance and taste of your water.

SPECIAL NOTICE - Required language for ALL community public water supplies: You may be more vulnerable than the general population to certain microbial contaminants, such as Cryptosporidium, in drinking water.  Infants, some elderly or immunocompromised persons such as those undergoing chemotherapy for cancer; those who have undergone organ transplants; those who are undergoing treatment with steroids; and people with HIV/AIDS or other immune system disorders can be particularly at risk from infections. You should seek advice about drinking water from your physician or health care provider. Additional guidelines on appropriate means to lessen the risk of infection by Cryptosporidium are available from the Safe Drinking Water Hotline at 1.800.426.4791.

PUBLIC PARTICIPATION OPPORTUNITIES - If you would like to talk to an Aqua Texas representative about your Water Quality Report, please call us at 1.877.987.2782, write us, or visit our website at AquaAmerica.com.  For more information from the EPA, you may call the U.S. Environmental Protection Agency Safe Drinking Water Hotline 1.800.426.4791.

En Español - Este informe incluye información importante sobre el agua potable. Si tiene preguntas o comentarios sobre este informe en español, favor de llamar al tel. 1.254.776.1999  para hablar con una persona bilingüe en español.

WEST BRAZOS WSC									         	 	              TX0730021

DEFINITIONS

Maximum Contaminant Level (MCL)

The highest permissible level of a contaminant in drinking water.  MCLs are set as close as possible to MCLGs as feasible using the best available technology.

Maximum Contaminant Level Goal (MCLG) - The level contaminant in drinking water below which there is no or expected health risk. MCLGs allow a margin of safety.

Maximum Residual Disinfectant Level (MRDL) - The highest level of disinfectant allowed in drinking water.  There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

Maximum Residual Disinfectant Level Goal  (MRDLG)  -  The level of a drinking water disinfectant below which there is no known or expected risk to health.  MRDLGs do not reflect the benefits of the use of a disinfectant to control microbial contamination.

Treatment Technique (TT) A required process intended to reduce the level of a contaminant in drinking water.

Action Level (AL) - The concentration of a contaminant which, if exceeded triggers treatment or other requirements that a water system must follow.

ppm - parts per million, or milligrams per  liter (mg/l)

ppb - parts per billion, or micrograms per liter (µg/L)

ppt - parts per trillion, or nanograms per liter

ppq - parts per quadrillion, or picograms per liter

NTU -Nephelometric Turbidity Units

MFL - million fibers per liter (a measure of asbestos)

pCi/L - picocuries per liter (a measure of radioactivity)

About the following pages - The pages that follow list all of the federally regulated or monitored contaminants which have been found in your drinking water. The U.S. EPA requires water systems to test for up to 97 contaminants.

Maximum Residual Disinfectant Level

| Year (Range) | Disinfectant | Average Level | Minimum Level | Maximum Level | MRDL | MRDLG | Unit of Measure | Source of Disinfectant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Free Chlorine Residual | 1.81 | 1.0 | 3.2 | 4 | 4 | ppm | Disinfectant used to control microbes |

Disinfection Byproducts

| Year | Contaminant | Average Level | Minimum Level | Maximum Level | MCL | MCLG | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Total Trihalomethanes | 12 | 12 | 12 | 80 | NA | ppb | Byproduct of drinking water disinfection |

Unregulated Initial Distribution System

Evaluation for Disinfection Byproducts - 	WAIVED OR NOT YET SAMPLED

Lead and Copper

| Year (Range) | Contaminant | The 90th Percentile | Number of Sites Exceeding Action Level | MCLG | Action Level | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Lead | 1.73 | 0 | 0 | 15 | ppb | Corrosion of household plumbing |
| 2023 | Copper | 0.136 | 0 | 1.3 | 1.3 | ppm |  |
| If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. Aqua is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your cold water tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead. |  |  |  |  |  |  |  |

WEST BRAZOS WSC									         	 	              TX0730021

Revised Total Coliform Rule (RTCR) - Coliform Bacteria (Level 1 Assessment Triggered)

| Maximum Contaminant Levels (MCL) Goal | Highest Number of Monthly Positive Coliform Samples | Total Number of Positive E. coli Sample(s) | Maximum Contaminant Level (MCL) Fecal or E. Coli | E. coli Violation | Positive Sample Collected | Repeat Samples | Likely Sources of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3 | 0 | A routine sample and a repeat sample are total coliform positive, and one is also E. coli positive. | NO | May 3, 2023 | May 5, 2023 | Naturally present in the environment |
| Coliforms are bacteria that are naturally present in the environment and are an indicator that other, potentially harmful, waterborne pathogens may be present, or that a potential pathway exists through which contamination may enter the drinking water distribution system. Because we had 2 total coliform positive samples in May 2023, we triggered a level 1 assessment. This required assessment looks for potential problems in water treatment or within the distribution system. The Level 1 assessment was completed and identified the most probable cause of the positives; and Aqua implemented a corrective action that prevented any other additional problems or positives. We will continue to identify improvements to the operation of this PWS. All other samples collected for the year were free of bacteria. |  |  |  |  |  |  |  |

Inorganic Contaminants

| Year (Range) | Contaminant | Average Level | Minimum Level | Maximum Level | MCL | MCLG | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021 & 2023 | Arsenic | 5.7* | 4.1 | 8.6* | 10 | 0 | ppb | Erosion of natural deposits |
| 2021 & 2023 | Barium | 0.082 | 0.076 | 0.086 | 2 | 2 | ppm |  |
| 2021 & 2023 | Chromium | 7.1 | 5.2 | 8.5 | 100 | 100 | ppb | Discharge from steel and pulp mills; Erosion of natural deposits. |
| 2021 & 2023 | Fluoride | 1.4 | 1.07 | 2.06 | 4 | 4 | ppm | Erosion of natural deposits; water additive which promotes strong teeth |
| 2023 | Nitrate | 0.0559 | 0.0486 | 0.0662 | 10 | 10 | ppm | Runoff from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits |
| *REQUIRED ADDITIONAL HEALTH INFORMATION FOR ARSENIC - Because the highest reported arsenic level on this report is between 5ppb and 10 ppb, this information is required by EPA: "While your drinking water meets EPA's standard for arsenic, it does contain low levels of arsenic. EPA's standard balances the current understanding of arsenic's possible health effects against the costs of removing arsenic from drinking water. EPA continues to research the health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems." |  |  |  |  |  |  |  |  |

Radioactive Contaminants

| Year (Range) | Contaminant | Average Level | Minimum Level | Maximum Level | MCL | MCLG | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021 | Combined Radium 226 & 228 | 1.5 | 1.5 | 1.5 | 5 | 0 | pCi/L | Erosion of natural deposits |

Unregulated Contaminants- 	NOT REPORTED OR NONE DETECTED

Organic Contaminants-		TESTING WAIVED, NOT REPORTED, OR NONE DETECTED

Ground Water Rule (GWR) - 		REPORTED MONTHLY TESTS FOUND NO E. coli

WEST BRAZOS WSC									         	 	              TX0730021

Mandatory Public Notification Language Secondary FLUORIDE Violation West Brazos WSC Public Water System ID # 0730021 This is an alert about your drinking water and a cosmetic dental problem that might affect children under nine years of age. At low levels, fluoride can help prevent cavities, but children drinking water containing more than 2 milligrams per liter (mg/l) of fluoride may develop cosmetic discoloration of their permanent teeth (dental fluorosis). The drinking water provided by your community water system, West Brazos WSC, had a maximum fluoride concentration of 2.06 mg/l. Dental fluorosis, in its moderate or severe forms, may result in brown staining and/or pitting of the permanent teeth. This problem occurs only in developing teeth, before they erupt from the gums. Children under nine should be provided with alternative sources of drinking water or water that has been treated to remove the fluoride to avoid the possibility of staining and pitting of their permanent teeth. You may also want to contact your dentist about proper use of fluoride-containing products by young children. Drinking water containing more than 4 mg/l of fluoride (the U.S. Environmental Protection Agency’s drinking water standard) can increase your risk of developing bone disease. Your drinking water does not contain more than 4 mg/l of fluoride, but we’re required to notify you when we discover that the fluoride levels in your drinking water exceed 2 mg/l because of this cosmetic dental problem. For more information, please call the Water Compliance Coordinator of Aqua Texas at 512.990.4400 (X56109). Some home water treatment units are also available to remove fluoride from drinking water. To learn more about available home water treatment units, you may call NSF International at 1.877.NSF.HELP.

CUSTOMER RESPONSIBILITIES - Our water systems are designed and operated to deliver water to our customers’ plumbing systems that complies with state and federal drinking water standards. This water is disinfected using chlorine, but it is not necessarily sterile. Customers’ plumbing, including treatment devices, might remove, introduce, or increase contaminants in tap water. All customers, and in particular operators of facilities like hotels and institutions serving susceptible populations (like hospitals and nursing homes), should properly operate and maintain the plumbing systems in these facilities. You can obtain additional information about these matters from the EPA's Safe Drinking Water Hotline at 1.800.426.4791.